Welcome to my semester 4 university project! This project is my individual effort, and it comes with several important features:

- Main Power Control: There's a primary switch that you can use to turn the product on and off.

- Multimeter and Power Supply Modes: Another switch allows you to switch between two modes: multimeter and 5V power supply.

- User-Friendly Controls: You'll find two rotary switches designed for mode selection and adjusting the ohmmeter's scale.

- Readings Display: All the important readings are conveniently displayed on an LED screen.

- Rechargeable Battery: What makes this product special is its built-in rechargeable battery system. Say goodbye to constant battery replacements; this product includes an integrated charger mechanism.

# Specifications

- This product is mainly a multimeter. Also this can be used as a 5V DC power supply.

- Multimeter has four modes.

- Voltmeter

  - Measurement range = 0V to 50V

- Ammeter

  - Measurement range = 1mA to 2A

- Ohmmeter

  - Measurement range = 0 ohm to 1M ohm

  - There are four resistance scales.

  - 0 to 1K ohm

  - 1K to 100K ohm

  - 100K to 400K ohm

  - 400K to 1M ohm

- Continuity checker

- Probes do not have to change when it is working as an ammeter.

- This product has a charging feature.

- Once it is fully charged, it can be used 12 hours straight.

- There is a two way switch to change the mode between multimeter and power supply.

- weight = 500g

- Dimensions = 13cm × 9cm × 2.5cm

What you can find here :

- Component list

- Block Diagram

- Circuit diagrams

- Schematics Designs

- PCB Designs

- Enclosure Design

For more information, feel free to explore all the files and data in my repository. I've included comprehensive documentation, source code, and additional resources for a deeper dive into this project. Your feedback and contributions are highly appreciated.

Component List

- Atmega328p Microcontroller

- 3P4T Rotary Switches

- 16*2 LCD Display

- Single Pole Double Throw Switch

- On-off Switch

- 1200mAh LiPo Battery

- TP4056 Charging module

- MT3608 Boost Converter

Block Diagram

Circuit Diagrams

Schematics Design

Sheet 1 of 3

Sheet 2 of 3

Sheet 3 of 3

PCB Designs

Enclosure Designs